DiagIoT

System Technical Summary

Version 1.0

Date: 2026-08-28

Classification: Internal / Product Technical Reference

# 1. Executive Overview

DiagIoT is an agentic drift detection platform for embedded IoT. It autonomously monitors fleets of embedded devices, detects when their behavior “drifts” from known-good baselines, and orchestrates investigation, reproduction, and remediation through three specialized autonomous agents.

## Core Problem

Embedded IoT devices in factory, logistics, energy, and infrastructure deployments degrade silently. A sensor that once read within ±2 LSB starts reading ±8 LSB. A firmware update subtly changes timing behavior. A power rail develops ripple that couples into ADC channels. These drifts are slow, cross-domain, and invisible to simple threshold monitors — yet they cause cascading failures in the field.

## DiagIoT's Answer

Instead of relying on static rules or manual diagnosis, DiagIoT deploys three autonomous agents that each own a phase of the device lifecycle:

| Agent | Phase | Mission |
| --- | --- | --- |
| 🛡 Pre-Ship Agent | Pre-deployment | Gate-keep firmware before it reaches devices — scan, score, and block releases that carry drift risk |
| 👁 Monitor & Investigate Agent | In-field | Continuously watch the fleet, detect drift, correlate across signals, form root-cause hypotheses, and replay incidents |
| 🧭 Onboarding Agent | Human enablement | Guide new engineers through the platform, curate institutional knowledge, and accelerate investigation skills |

Together, these agents create a closed loop: scan before ship → monitor in field → investigate when drift is found → capture knowledge → apply it to the next scan.

# 2. System Architecture

## 2.1 High-Level Topology

┌─────────────────────────────────────────────────────────────────┐ │ DiagIoT Platform │ │ │ │ ┌─────────────┐ ┌───────────────────────┐ ┌──────────────┐ │ │ │ Pre-Ship │ │ Monitor & │ │ Onboarding │ │ │ │ Agent │ │ Investigate Agent │ │ Agent │ │ │ │ │ │ │ │ │ │ │ │ • Scan │ │ • Field Monitoring │ │ • Onboarding │ │ │ │ • Score │ │ • Drift Detection │ │ • Knowledge │ │ │ │ • Block │ │ • Correlation │ │ Curation │ │ │ │ • Baseline │ │ • Root-Cause │ │ • Skill │ │ │ │ Capture │ │ Hypothesis │ │ Building │ │ │ └──────┬──────┘ └───────────┬───────────┘ └───────┬───────┘ │ │ │ │ │ │ │ └──────────────────────┼───────────────────────┘ │ │ │ │ │ ┌─────────▼─────────┐ │ │ │ Shared Services │ │ │ │ │ │ │ │ • Device Registry │ │ │ │ • Drift Engine │ │ │ │ • Alert Bus │ │ │ │ • Firmware Store │ │ │ │ • Knowledge Base │ │ │ │ • Integration Hub │ │ │ └─────────┬──────────┘ │ │ │ │ │ ┌────────────────┼────────────────┐ │ │ ▼ ▼ ▼ │ │ ┌───────────────┐ ┌──────────────┐ ┌───────────────┐ │ │ │ CI/CD │ │ IDEs / │ │ Build │ │ │ │ Pipelines │ │ Tooling │ │ Systems │ │ │ └───────────────┘ └──────────────┘ └───────────────┘ │ └─────────────────────────────────────────────────────────────────┘ │ ┌────────────────┼────────────────┐ ▼ ▼ ▼ ┌───────────────┐ ┌──────────────┐ ┌───────────────┐ │ IoT Device │ │ IoT Device │ │ IoT Device │ │ Fleet │ │ Fleet │ │ Fleet │ └───────────────┘ └──────────────┘ └───────────────┘

## 2.2 Agent Communication Model

Agents are semi-autonomous — each owns its domain and acts independently, but they share a common data substrate and can feed each other:

Pre-Ship ──(baseline data)──► Monitor & Investigate Monitor & Investigate ──(incident findings)──► Onboarding (knowledge articles) Onboarding ──(runbooks)──► Pre-Ship (scan policies) Monitor & Investigate ──(field drift patterns)──► Pre-Ship (scan rule updates)

No agent commands another. Communication is event-driven and asynchronous:

- When the Pre-Ship Agent blocks a firmware, it publishes a block event → Monitor subscribes to track the device post-deployment.

- When the Monitor Agent resolves a root cause, it publishes a finding → Onboarding subscribes to create or update a knowledge article.

- When the Onboarding Agent curates a new runbook, it publishes a rule suggestion → Pre-Ship subscribes to update scan policies.

# 3. Agent Deep-Dives

## 3.1 Pre-Ship Agent 🛡

Identity: Teal #4ECDC4 · Shield icon 🛡

Phase: Pre-deployment

Scope: 1 dedicated screen (Pre-Ship Scan) + shared Dashboard

### Responsibilities

| Function | Description |
| --- | --- |
| Firmware Scanning | Accept firmware binaries from CI/CD pipelines, IDEs, or manual upload. Decompile/analyze for drift risk indicators. |
| Drift Scoring | Compute a drift risk score (0.00–1.00) per binary, comparing against known-good baselines captured from the field by the Monitor Agent. |
| Gate Decision | Block releases exceeding the configurable threshold (default 0.70). Flag releases in the 0.40–0.70 warning zone for human review. |
| Baseline Capture | When a firmware is explicitly marked as “good” (e.g., a stable release tag), record its behavioral signature as the new baseline for that device type. |
| Build Integration | Hook into build systems (Arduino IDE, Keil, VS Code, Jenkins, GitHub) to automatically scan on build completion. |

### Drift Scoring Model

The Pre-Ship Agent computes a composite drift risk score from multiple signal categories:

| Signal | Weight | Source |
| --- | --- | --- |
| Binary structural diff | 30% | Firmware diff against baseline |
| Behavioral signature | 25% | Runtime telemetry comparison |
| Known vulnerability patterns | 20% | Knowledge Base patterns from resolved incidents |
| Hardware compatibility flags | 15% | HW revision + component specification checks |
| Config drift | 10% | Compiler flags, optimization levels |

### Thresholds

- < 0.40 → Pass — auto-flash

- 0.40 – 0.70 → Warn — review required

- ≥ 0.70 → Block — exceeds safety threshold

### Pre-Ship Scan Screen

The scan screen is organized as a vertical pipeline (.pipe-flow):

┌───────────────┐ │ Source │ Where the firmware came from (GitHub, Jenkins, IDE…) ├───────────────┤ │ Binary │ The firmware artifact name + version ├───────────────┤ │ Baseline │ Known-good version being compared against ├───────────────┤ │ Drift Score │ 0.00–1.00 with color-coded gauge ├───────────────┤ │ Verdict │ Pass / Warn / Block with action button ├───────────────┤ │ Detail │ Breakdown of contributing factors └───────────────┘

Below the pipeline: a filterable scan history table (source dropdown, status dropdown, search input) showing all past scans with columns for time, source, description, binary, drift score, and verdict.

## 3.2 Monitor & Investigate Agent 👁

Identity: Blue #6C8CFF · Eye icon 👁

Phase: In-field

Scope: 5 dedicated screens (Field Monitor, Drift Analysis, Event Timeline, Firmware Diff, Reproduce Incident) + shared Dashboard

### Responsibilities

| Function | Description |
| --- | --- |
| Fleet Monitoring | Continuously ingest telemetry from all devices in the fleet. Surface health status (Healthy / Drifting / Critical) per device type. |
| Drift Detection | Compute per-device drift scores in real time. Alert when scores cross configurable thresholds. |
| Cross-Signal Correlation | Identify relationships between drift signals across subsystems (e.g., power ripple ↔ ADC offset). Build a correlation matrix across all signal pairs. |
| Root-Cause Hypothesis | From correlated signals, generate ranked hypotheses (Primary / Secondary / Alternative) with confidence scores, traceable to specific hardware components and firmware versions. |
| Incident Replay | Load a field incident into the simulation engine. Inject the hypothesized fault conditions. Compare simulation output to field readings. Validate or reject the root-cause hypothesis. |
| Firmware Diff | Binary-level comparison between the running firmware version and the baseline. Highlight structural changes, added/removed functions, and register-level diffs. |
| Event Timeline | Chronological log of all fleet events (drift alerts, firmware pushes, config changes, hardware incidents) with severity-colored markers. |

### Drift Detection Pipeline

Device Telemetry │ ▼ Ingest ─► Feature Extraction ─► Baseline Comparison │ ▼ Drift Score (0.00–1.00) │ ├── Score < 0.40 → Healthy (no action) │ ├── Score 0.40–0.70 → Drifting │ │ │ ▼ │ Alert Bus → Correlation Engine │ └── Score ≥ 0.70 → Critical │ ▼ Urgent Alert + Auto-Investigate │ ▼ Correlation Engine │ ▼ Root-Cause Hypotheses │ ▼ Incident Replay (if hypothesis validated)

### Screen Breakdown

Field Monitor — Fleet-wide health dashboard:

- System registry (5 device types with device counts + health status)

- Recent alerts table with Agent column, drift score, severity badge, and device/time/type columns

- Live indicator with pulse animation

- Filter bar (search + status dropdown + agent filter)

Drift Analysis — Deep signal investigation:

- 6×6 correlation matrix (signal vs signal, values 0.00–1.00)

- Primary root-cause path highlighted (ADC ↔ Power = 0.79)

- 3 ranked root-cause hypotheses with confidence scores

- Hypothesis cards: Primary (🔴 89%), Secondary (🟡 62%), Alternative (🔵 31%)

- Each hypothesis links to hardware components (C34, U5) and firmware versions

Event Timeline — Chronological fleet log:

- Vertical timeline with colored severity dots (red = critical, amber = warning, blue = info)

- Each event: timestamp, device, event type, description

- Filter bar for time range and severity

Firmware Diff — Binary comparison:

- Side-by-side code view (old vs new)

- Diff highlighting: .diff-add (green), .diff-del (red)

- Register-level changes annotated

- Link back to drift analysis for correlated signals

Reproduce Incident — Simulation lab:

- Simulation setup: incident selector dropdown + load button

- Process flow: Load Baseline → Inject Drift → Monitor Response → Compare to Field → Validated ✓

- Simulation parameters: target device, FW version, HW revision, injected fault, CPU load, temperature

- Reproduction results: match score (96%), simulated vs field drift comparison, confirm/adjust buttons

## 3.3 Onboarding Agent 🧭

Identity: Amber #F0A030 · Compass icon 🧭

Phase: Human enablement

Scope: 2 dedicated screens (Engineer Onboarding, Knowledge Base) + shared Dashboard

### Responsibilities

| Function | Description |
| --- | --- |
| Guided Onboarding | Walk new engineers through the platform's investigation workflow via interactive modules. Track progress per module and per engineer. |
| Knowledge Curation | Automatically create and update knowledge base articles when the Monitor Agent resolves incidents. Tag articles by severity and device type. |
| Skill Assessment | Verify engineers understand drift detection concepts through hands-on lab exercises with real (anonymized) incident data. |
| Contextual Help | Provide in-screen guidance during active investigations, referencing relevant past incidents and runbooks. |

### Onboarding Modules

| Module | Type | Progress | Content |
| --- | --- | --- | --- |
| System Architecture | Structured lesson | ✅ Complete | Platform overview, agent roles, data flows |
| Drift Concepts | Interactive tutorial | 65% (4/6) | What drift is, scoring model, threshold interpretation |
| Hands-On Lab | Practical exercise | 20% (1/5) | Guided investigation of real anonymized incidents |

### Knowledge Base

Articles are curated by the Onboarding Agent based on field findings:

| Category | Examples |
| --- | --- |
| Critical incidents | Power-Rail Ripple → ADC Coupling, Clock Skew in Multi-MCU Systems |
| Warning patterns | Memory Leak Patterns in Bare-Metal FW |
| Runbooks / Guides | Pre-Ship Scan Runbook, Interrupt Vector Priority Guide |
| Post-mortems | Factory Line A Post-Mortem (Resolved) |

Each article card shows: title, severity badge, device type, update timestamp, and tags. Articles are auto-updated when new drift patterns are detected — the Onboarding Agent revises content weekly based on Monitor Agent findings.

# 4. Data Model

## 4.1 Core Entities

### Device

Device ├── device_id: string (e.g., "SensorHub-X4 #3192") ├── device_type: enum (SensorHub-X4, ThermoNode-R2, │ ActuatorBridge-M7, EdgeGateway-E1, PowerCtrl-Z8) ├── firmware_version: string ├── hardware_revision: string ├── health_status: enum (Healthy, Drifting, Critical) ├── drift_score: float (0.00–1.00) └── last_seen: timestamp

### Firmware

Firmware ├── artifact_name: string (e.g., "fw-sensor-v2.4.1.hex") ├── version: string ├── source: enum (Arduino IDE, GitHub, VS Code, Keil, Jenkins, Docker) ├── drift_score: float (0.00–1.00) ├── verdict: enum (Pass, Warn, Block) ├── baseline_version: string └── scan_timestamp: timestamp

### Alert

Alert ├── alert_id: string ├── device: reference → Device ├── alert_type: string (e.g., "GPIO Register Mismatch") ├── drift_score: float ├── severity: enum (Critical, Warning, Info) ├── owning_agent: enum (Pre-Ship, Monitor) ├── timestamp: datetime └── status: enum (New, Investigating, Resolved)

### Incident

Incident ├── incident_id: string ├── affected_devices: list → Device ├── root_cause_hypotheses: list → Hypothesis ├── correlation_matrix: Signal[][] → float ├── simulation_result: SimResult (nullable) ├── status: enum (Open, Investigating, Reproduced, Resolved) └── created_at: timestamp

### Hypothesis

Hypothesis ├── rank: enum (Primary, Secondary, Alternative) ├── confidence: float (0.00–1.00) ├── description: string ├── hardware_components: list → string ├── firmware_versions: list → string └── validated: boolean

### KnowledgeArticle

KnowledgeArticle ├── article_id: string ├── title: string ├── severity: enum (Critical, Warning, Guide, Resolved) ├── device_types: list → string ├── content: markdown ├── curated_by: Onboarding Agent ├── last_updated: timestamp └── auto_updated: boolean

### Integration

Integration ├── name: string (e.g., "Arduino IDE") ├── type: enum (IDE, CI/CD, Build Tool, Debugger) ├── status: enum (Live, Connected, Partial, Disconnected) ├── connected_at: timestamp └── config: JSON

## 4.2 Drift Score Semantics

The drift score is the unifying metric across the entire system:

| Range | Label | Color | Action |
| --- | --- | --- | --- |
| 0.00–0.20 | Safe | #4ECDC4 (teal/green) | No action |
| 0.21–0.39 | Low | #6C8CFF (blue) | Informational |
| 0.40–0.69 | Warning | #F0A030 (amber) | Review required |
| 0.70–1.00 | Critical | #EF4444 (red) | Block / Urgent alert |

The same scale is used by both the Pre-Ship Agent (firmware scoring) and the Monitor Agent (device health scoring), ensuring a consistent risk language across the platform.

# 5. Integration Layer

DiagIoT connects to the tools that embedded teams already use:

## 5.1 Supported Integrations

| Integration | Type | Status | Hook Point |
| --- | --- | --- | --- |
| Arduino IDE | IDE | Live | Post-build scan trigger |
| GitHub | VCS | Live | PR push + release tag events |
| VS Code + PlatformIO | IDE | Live | Extension-based build hook |
| Keil / STM32Cube | IDE | Partial | Post-build hook (ARM toolchain) |
| Jenkins CI | CI/CD | Connected | Build completion webhook |
| Docker / OCI | Container | Connected | Image build scan |
| GitLab | VCS | Disconnected | Planned: merge request hooks |
| SEGGER Ozone | Debugger | Disconnected | Planned: flash + debug session hooks |

## 5.2 Integration Flow

Developer commits code │ ▼ CI/CD Pipeline triggers build │ ▼ Build produces firmware artifact ├──────────────────────────────────┐ ▼ ▼ Pre-Ship Agent scans artifact Manual upload │ (IDE / drag-drop) ▼ │ Drift Score computed │ │ │ ▼ ▼ Verdict rendered ◄────────────────────────────┘ Pass → Auto-flash Warn → Notify team Block → Halt pipeline Info → Log only

# 6. Interaction Model

## 6.1 User Personas

| Persona | Primary Screens | Typical Flow |
| --- | --- | --- |
| Firmware Engineer | Pre-Ship Scan, Firmware Diff | Commit → Build → Check scan verdict → Fix or release |
| Site Reliability Engineer | Field Monitor, Drift Analysis, Event Timeline | Receive alert → Investigate correlation → Form hypothesis |
| Field Engineer | Reproduce Incident, Knowledge Base | Load incident → Reproduce in lab → Validate fix |
| New Team Member | Onboarding, Knowledge Base | Complete modules → Read articles → Hands-on lab |
| Platform Admin | Dashboard, Integrations | Monitor fleet health → Configure integrations → Set thresholds |

## 6.2 Cross-Agent Workflow

A typical end-to-end investigation follows this pattern:

- Monitor Agent detects drift on ActuatorBridge-M7 #5501

- Alert surfaces on Field Monitor screen

- Engineer opens Drift Analysis → correlation matrix shows ADC ↔ Power = 0.79

- Monitor Agent generates 3 hypotheses → Primary: C34 capacitor under-specified

- Engineer opens Reproduce Incident → loads simulation

- Simulation confirms 96% match → root cause validated

- Monitor Agent publishes resolved incident to knowledge base

- Onboarding Agent auto-creates article: “Power-Rail Ripple → ADC Coupling”

- Pre-Ship Agent updates scan policy: flag C34-related firmware changes

- Next firmware scan catches the pattern pre-ship → block

# 7. Agent Technical Details

## 7.1 Agent Runtime Model

Each agent operates as an autonomous service with:

| Property | Description |
| --- | --- |
| Ownership | Each agent owns specific screens and the data flows within them |
| Independence | Agents act without requiring human trigger (monitor is always-on, pre-ship scans on build, onboarding curates on incident) |
| Event subscription | Agents subscribe to the shared Alert Bus and act on relevant events |
| State | Each agent maintains its own state (scan history, alert queue, module progress) |
| Identity | Agents present themselves to users with a consistent name, icon, and color across every touchpoint |

## 7.2 Agent Identity System

The identity system is designed so that users always know which agent is acting:

| Touchpoint | Pre-Ship | Monitor | Onboarding |
| --- | --- | --- | --- |
| Sidebar group label | 🛡 Pre-Ship Agent (teal) | 👁 Monitor & Investigate Agent (blue) | 🧭 Onboarding Agent (amber) |
| Sidebar active state | Teal left border + dim bg | Blue left border + dim bg | Amber left border + dim bg |
| Topbar pill | 🛡 Pre-Ship (teal, pulsing) | 👁 Monitor (blue, pulsing) | 🧭 Onboarding (amber, pulsing) |
| Dashboard status card | Teal top bar, scan metrics | Blue top bar, alert metrics | Amber top bar, module metrics |
| Screen subtitle tag | 🛡 Pre-Ship Agent (teal pill) | 👁 Monitor Agent (blue pill) | 🧭 Onboarding Agent (amber pill) |
| Alert attribution | 🛡 Pre-Ship (teal) | 👁 Monitor (blue) | — |
| Knowledge attribution | — | — | Curated by Onboarding Agent (amber banner) |
| Chat label | — | — | Onboarding Agent (amber text) |

## 7.3 Color Architecture

:root { --preship: #4ecdc4; /* Teal — Pre-Ship Agent base */ --preship-dim: rgba(78,205,196,0.12); --preship-border: rgba(78,205,196,0.35); --monitor: #6c8cff; /* Blue — Monitor Agent base */ --monitor-dim: rgba(108,140,255,0.12); --monitor-border: rgba(108,140,255,0.35); --onboard: #f0a030; /* Amber — Onboarding Agent base */ --onboard-dim: rgba(240,160,48,0.12); --onboard-border: rgba(240,160,48,0.35); }

Each agent has 3 tiers: base (full saturation), dim (12% opacity for backgrounds), and border (35% opacity for outlines). This creates a visual hierarchy where:

- Base = active/selected state (active tab, active sidebar link)

- Dim = idle/presentational background (pills, tags, card backgrounds)

- Border = containment outlines (pill borders, tag borders, banner borders)

# 8. Screen Inventory

## 8.1 Screen Ownership Map

| Screen | Agent | Primary Content |
| --- | --- | --- |
| System Health Dashboard | Shared (neutral) | Fleet KPIs, system registry, recent alerts, agent status panel |
| Pre-Ship Scan | 🛡 Pre-Ship | Scan pipeline, drift gauge, filterable scan history |
| Field Monitor | 👁 Monitor | Device health grid, live alerts table, filter bar |
| Drift Analysis | 👁 Monitor | Correlation matrix, root-cause hypotheses, signal paths |
| Event Timeline | 👁 Monitor | Chronological event log with severity markers |
| Firmware Diff | 👁 Monitor | Side-by-side binary comparison with diff highlighting |
| Reproduce Incident | 👁 Monitor | Simulation setup, parameter controls, reproduction results |
| Integrations | Shared (neutral) | Connected tool cards with status indicators |
| Engineer Onboarding | 🧭 Onboarding | Module progress cards, interactive chat panel |
| Knowledge Base | 🧭 Onboarding | Article grid with curation banner, severity tags |

## 8.2 Navigation Model

- Sidebar (fixed left, 240px): 4 branded groups with agent-colored headers

- Screen tabs (top of main area): 10 horizontal tabs, active state colored by owning agent

- Cross-linking: screens reference each other (e.g., Drift Analysis links to Firmware Diff, Reproduce Incident links back to Drift Analysis)

# 9. UI Framework

## 9.1 Design Tokens

| Token | Value | Usage |
| --- | --- | --- |
| --bg | #0F1117 | Page background |
| --surface | #181B24 | Card/container background |
| --surface2 | #1E2230 | Elevated surface (inputs, secondary panels) |
| --text | #E4E6F0 | Primary text |
| --text-dim | #7C809A | Secondary/muted text |
| --accent | #6C8CFF | Primary accent (same as Monitor Agent) |
| --border | #2A2E3D | Borders, dividers |
| --ok | #4ECDC4 | Success / safe / pass |
| --warn | #F0A030 | Warning / moderate risk |
| --danger | #EF4444 | Critical / blocked / fail |

## 9.2 Component Library

| Category | Components |
| --- | --- |
| Layout | Topbar (48px), Sidebar (240px), Main (scrollable), Frame label (fixed) |
| Navigation | Screen tabs, Sidebar groups + links, Agent pills |
| Data Display | Cards, Stat blocks, Data tables, Correlation matrix, Timeline |
| Visualization | Bar charts, Drift meters, Progress bars, Process flows, Pipeline diagrams |
| Code | Code blocks, Diff views (add/del highlighting) |
| Input | Filter bars (search + dropdown), Select dropdowns, Toggle switches, Buttons |
| Status | Severity badges (ok/warn/danger/info), Pulse dots, Live indicators, Status dots |
| Agent | Agent cards, Agent tags, Curation banners, Agent pills, Group labels |
| Onboarding | Module cards, Chat panels, Progress tracks |
| Integration | Integration cards (icon + status + config) |

## 9.3 Animation Inventory

| Animation | Duration | Purpose |
| --- | --- | --- |
| agent-pulse | 1.4s infinite | Agent status dot breathing (opacity + scale) |
| pulse-ring | 1.5s infinite | Live indicator expanding ring |
| Bar entrance | Staggered | Chart bars animate from 0% to target height on load |

# 10. Security & Compliance Model

## 10.1 Access Control

- Agent actions are audited: every scan, alert, hypothesis, and knowledge article is timestamped and attributed to the responsible agent.

- Block decisions by the Pre-Ship Agent can be overridden by authorized engineers with documented justification.

- Knowledge Base articles created by the Onboarding Agent are reviewable before publication in production environments.

## 10.2 Data Handling

- Device telemetry is processed in-stream; raw signals are retained per configurable retention policy.

- Firmware binaries are stored in the Firmware Store with integrity checksums.

- Drift scores are non-repudiable: once computed, they are logged and cannot be retroactively modified.

# 11. Scalability & Deployment

## 11.1 Fleet Scale

- Current design targets fleets of 100–10,000 devices.

- Device registry supports grouping by type, location, firmware version, and health status.

- Correlation engine processes up to 36 signal pairs (6×6 matrix) per investigation.

## 11.2 Integration Extensibility

New integrations follow the Integration Card pattern:

- Register the tool name, type, and icon

- Implement the webhook/polling connector

- Map build/commit events to Pre-Ship scan triggers

- Surface status (Live / Connected / Partial / Disconnected) in the Integrations grid

# 12. Glossary

| Term | Definition |
| --- | --- |
| Drift | A measurable deviation in device behavior from a known-good baseline |
| Drift Score | A normalized 0.00–1.00 metric quantifying the magnitude of drift |
| Baseline | The behavioral signature of a firmware version known to be good in the field |
| Root-Cause Hypothesis | An agent-generated explanation for observed drift, ranked by confidence |
| Correlation Matrix | A signal×signal heatmap showing cross-domain drift relationships |
| Incident Replay | Simulating field conditions in a lab to verify a root-cause hypothesis |
| Agent | An autonomous service that owns a domain of the DiagIoT workflow |
| Curation | The Onboarding Agent's process of creating/updating knowledge articles from incident data |
| Gate Decision | The Pre-Ship Agent's Pass/Warn/Block verdict on a firmware release |

# 13. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-08-28 | DiagIoT Team | Initial system technical summary |

End of document.